Timingconstrained ROC Firmware

This document reports on features of the ROC Firmware released on 05/04/2025 to “timingconstrained” GIT branch of the ROC_FW project. It is a project in Libero v2024.2 after Richie made some changes and improvement to meet timing (see Appendix A for details). See here for a list of changes. It is divided in three part:

- new features: these are features first available in this ROC_FW release

  - Programmable readout timeout

  - DDR Dump

  - RS485 Interface

  - Fine fiber synchronization

  - DIGI SPI read/write via fiber

- disabled features: these are features existing in previous release and now disabled to achieve timing closure

  - Error Counters

  - DIGI Serdes Simulation

- old features: these are features already existing in previous versions of the firmware, and reported here for purpose of documentation

  - NEWRUN and HALTRUN_EN option

  - Read of local event tag in DIGI

  - Clock and Event Window controls via serial vs fiber

  - Loopback logic

New features:

Programmable readout timeout: the readout timeout is the maximum time allowed to service a Data Request (DREQ) from the DTC, ie the time from the DREQ being received and the data being fully read from the DDR and dumped in one of the two EVT_FIFOs to be sent back to the DTC.

For an event with a large number of hits, this time can be very long as it requires the DIGI data to arrive at the ROC and be completely flushed from all of the 4 lanes, even if the data saved for the event is only the first 8kB received. Only then the event is written to DDR, recorded in the DREQ_FIFO, and finally read back from DDR into one of the two EVT_FIFOs.

This readout timeout, EVENT_TIMEOUT[19:0], is in units of 12.5 ns (or 80 MHz clock ticks) and is set by a DCS write to address=60 (for EVENT_TIMEOUT[15:0]) and address=61 (for EVENT_TIMEOUT[19:0]). Its default value at power up is 0xFFFF, or 820us.

It can be read via a DCS read of address=60 (for EVENT_TIMEOUT[15:0]) and address=61 (for EVENT_TIMEOUT[19:0]).

NB: In previous version of the ROC firmware this timeout was hardwired to a value of 0x3FFF or 205us.

DDR Dump:  Ability to access DDR memory content at a programmable MEM_OFFSET[19:0].

Data from each window tag is written to the DDR in multiple of 64-bit at the boundary of 1KB block, and starts with two 64-bit event headers (more about that below). The remaining 126x64b will contain hit data. The DDR4 memory installed on the ROC is 8Gb and each address contains 8 bit. Hence there are a total of 2**20 x 1KB blocks, each spanning 0x400 addresses. A given memory offset thus correspond to the physical address MEM_OFFSET*0x400. Upon issuing the MEM_START command, 1KB of memory is dumped to an internal DUMP_FIFO which has a 64bx128 write port and a 16bx512 for read port. The DUMP_FIFO can be read via a DCS Block read of 512 words from address 512(0x200).

Recommended commands sequence for a DDR dump

- Check that DUMP_FIFO is empty, ie that a DCS read of address=20 returns 0x0 (where bit[15]=DUMP_FIFO full; bit[7:0]=DUMP_FIFO WRCNT in units of 64-bit words) and a DCS read of address=21 returns 0x8000 (where bit[15]=DUMP_FIFO empty and bit[9:0]= RDCNT, in units of 16-bit words). If otherwise, issue DDR reset via DCS write of any value to address=14.

- Set MEM_OFFSET to read via DCS write of value MEM_OFFSET[15:0] to address=33 and value MEM_OFFSET[19:16] via address=34. Default value after power up of MEM_OFFSET is zero.

- Issue MEM_READ via DCS write of value=1, followed by 0, to address=32.

- Check that DDR dump is ready to be read by requesting that DCS read of address=20 be equal to 0x8080  and read of address=21 be equal to 0x200.

- Read the DUMP_FIFO by issuing a DCS Block Read of N=512 words to address 512(0x200)

The DDR is written to a monotonically increasing addresses and starts at an offset = 0 after a ROC FPGA reflash OR a board power cycle OR for the first event tag after a NULL Heartbit. Once the 4Gb DDR memory is exhausted, at memory block 1,048,575 (2**20-1 ), it will wrap-around and starts again from an offset=0. Data from a single event is not allowed to straddle the memory wrap-around point. The start address (ie the first 1kB block MEM_OFFSET) of the last event read from DDR, or DDR_READ_ADDR, is saved to address=15 (DDR_READ_ADDR[15:0]) and address=16 (DDR_READ_ADDR[19:16]).

When data from events are written to DDR, they are divided in 1 KB blocks  containing two 64-bit header words and a maximum of 126 x 64-bit payload words. If the event contains less than 126 x 64-bit, the remaining words in the block will be meaningless or contains info from a previous event block landing at that address. If the event contains more than 126 x 64b, the remaining payload words are written to the next address in memory. The maximum allowed event size is currently (2**10-1)x64b, or 8kB (ie 255 hits of 256b each). An event of size larger than this is labelled as overflow event and it will span 9x1kB blocks in order to fit the header words. For instance an event with 96 hits of 256-bit each, will have 384 x 64-bit word payload, and will require to span four blocks, the first three with 126 (0x7E) 64-bit-words each and the last one with 6.

The event header words contain information about the event tag and the payload to expect, specifically:

1st header word (bits not specified are set to zero):

- bit[61] = event error (set to 1 if any SerDes lane report data from different local tag  OR if any SerDes lane report the TAG_SYNC error bit. Such error happens when the SYNC_BIT signals sent by the ROC to the DIGI, indicating that the local tag counter rolls over the 2**14=8192 count, does not agree with the local tag counter in the DIGI)

- bit[60] = event overflow (set to 1 if event has more than maximum allowed size of 8KB. When this happen, the ROC firmware writes to DDR first the full data reported by the DIGI CAL0 lane, followed by CAL1, HV0 and HV1 lane, respectively. Once the maximum size is reached, data from the current and any remaining lanes are dropped.)

- bit[58:48] = remaining event size, ie number of words still to be written for the event, in unit of 64-bit (where 64-bit corresponds to ¼ of a hit size  or ½ of a DTC Data Request packet size)

- bit[47:0] = local event tag as offset tag (ie first tag of a new run after a null HB) plus local tag sent by DIGI.

2nd header word :

- bit[63:56] = number of 64-bit words in current block, ie a number from 0, when event has no hits, to maximum allowed of 126 (for instance if event has more than 31 x 256b hits)

- bit[55:48] = block number in event ie current 1kB block for event reuiring more than multiple blocks, starting from 1

- bit[47:0]   = heartbit event tag ie tag for the hearbit before event marker that starts the event window

For instance, let’s say that the 100th event of a run starting at event tag 1000 contain 96 hits. The event will have local tag 99 (0x63) since the local tag counter starts at zero, and offset tag 1000 (0x3E8), corresponding to event tag 1100 (0x44B). The event will contain 96x4=384(0x180) 64-bit words and spread over four blocks. The remaining event size at block 1 will be the full number of blocks in the event, followed by 384-126=258(0x102), 258-126=132(0x84), and 132-126=6, respectively.

The second header word will report the event tag from the heartbit before the event window tag starts, ie 1000+99, the block number of the data (1, 2, 3 and 4, respectively) and the number of payload words per block (126, 126, 126 and 6, respectively).

The first and  second header words for each of the four blocks is therefore expected to be:

1st header word		2nd header word

1st block: 	0x0180_0000_0000_ 044B	0x7E01_0000_0000_044B

2nd block:	0x0102_0000_0000_ 044B	0x7E02_0000_0000_044B

3rd block:	0x0084_0000_0000_ 044B	0x7E03_0000_0000_044B

4th block:	0x0006_0000_0000_ 044B	0x7E04_0000_0000_044B

Using this information, the user knows how many 64-bit word read back by the DCS Block contain true hit information, the remaining in the block just reporting stale data filled up in previous events.

The 48 LSB bits of the second header word are used for some sanity checks. During a memory write, they are compared with the 48 LSB of the first header word, and the number of times they do not agree is saved to a counter read via address=72. During a memory read, they are compared to the Data Request event tag, and the number of times they do not agree is saved to a counter read via address=73.

To have an idea of the memory location we are currently at, one can use a DCS read of address 15&16, which report the starting memory address for the last event read from DDR memory and sent to the DTC.

RS485 Interface: RS485 is a single master (computer) multi-slaves serial communication protocol. It uses balanced differential pair data transmission lines to provide a robust and noise-immune communication. It is meant to be the last-resort path between the DRAC and the outside world in case the fiber is not responding.

In case of the Mu2e tracker readout, it is implemented via a 8-wire flat ribbon cable connecting a dedicated header on the key board for up to 12 DRAC modules in a daisy-chain fashion. Each DRAC in the chain respond to a unique 9-bit address, which can be set via a dedicated serial register or read from the sNVM. The last DRAC in the chain is connected to the computer via a serial cable.

The protocol used for the tracker-to-outside world communication expects the master to send and receive 3 bytes. The ROC firmware (slave) receives and sends data via a  dedicated RS485_RX input pin and RS485_TX output pin. These signals are handled by the RS485Register VHDL module instantiated inside the SLOWCONTROLS block, which interfaces

the RS485 master to the RiskV microprocessor. This module expects changes of the RX/TX lines at a baud rate of 38400 on the 50 MHz clock, ie every 1302 clocks (or 26 usec).

The RS485_RX line is decoded by the “Receive_Process” state machine which upon detecting the RX signal low for at least 1302 x 50-MHz-clocks, looks at the next 3-bytes.

- 1st byte:  	bit[7]    =  DRAC_ADDRESS[8]

bit[6:0] = “101_1010” , which is the reserved code to identify a 			MASTER_TO_SLAVE command

- 2nd byte = DRAC_ADDRESS[7:0].

Together with the MSB bit of the first byte, this address uniquely identifies each DRAC in the tracker has to respond to the RS485 command. In the present implementation of the firmware, such address is set by a DIGI write of register 0xC3 (default value=0). In future implementation, it will be read from sNVM at power up and will correspond to the Minnesota PANEL_ID.

- 3rd byte = RS485_COMMAND

This is a unique command sent to the RiskV uProc together with the RX_READY signal. Upon seeing the RX_READY signal, the uProc decode the command and set the RX_ACK signal back to the firmware, which concludes the MASTER_TO_SLAVE transaction.

After decoding the RS485_COMMAND, the uProc prepares 16 bits of REPLY_DATA to be sent back to the master. When ready, it issues a TX_START signal to the ROC firmware, which in turns starts the “Transmit_Process” state machine. This state machine is responsible for driving sending data back to the RS485 slave as a series of 3 bytes which are serialized to the RS485_TX line. The RS485 protocol expect these 3 bytes to provide the following information:

- 1st byte:  	0xEF which is a reserved code to identify SLAVE_TO_MASTER command

- 2nd byte: REPLY_DATA[7:0]

- 3rd byte: REPLY_DATA[15:8]

The uProc drives TX_START low right after the REPLY_DATA is ready thus concluding the SLAVE_TO_MASTER transaction. The ROC firmware can implement a programmable delay (in increments of 100 ms) before starting serializing the REPLY_DATA in case the master is not yet ready to receive data (ie while waiting for “set_rs485_receive” to be seen). This programmable delay has a default value of 1 (ie 100 ms) and can be change via a DIGI write of register 0xC4.

Fine Fiber synchronization: this features allows for a fine alignment of the DTC fiber data at the 5ns/20 = 250 ps level. It sets the 20-bit word encoding of the 0x3CBC character word used to lock the DTC fiber to one of its 20-possible values. Any other value, sampled during the fiber locking and alignment process, generated a reset of the fiber transceiver. Default value = 0x941CF. Alternative values: 0x941CF right_shifted by BITSLIP_COUNTER[4:0], where BITSLIP_COUNTER goes from 0 to 20.

This alignment procedure is driven by the TOP_SERDES/XCVR_Block/Clock_aligner.vhd module and control by DRAC register 1, which is a RD/WR register.

DCS write to address=1  sets BITSLIP_COUNTER and starts the alignment procedure.

DCS read of address=1 returns:

bit[15]=1 if alignment procedure was successful

bit[4:0]=BITSLIP_COUNTER

DIGI SPI read/write via fiber: this feature allows to send RD/WR commands to the DIGIs via fiber, bypassing the default interface provided by the RiskV microProcessor. It uses the fiber to drive signals directly to and from the TWI_Controller.vhd module implemented inside the SLOWCONTROLS/Registers block.  Separate signals are used to send commands to the CAL and to the DIGI FPGA.

DCS_DIGIRW_SEL: this is the enable that allows to bypass the uProc.  It can be set high via a DCS write to address=6. A DCS read of address=6 returns its current value. Default set to 0

DCS_CAL_INIT:  set to 1/0  via DCS write of address=23. Must toggle 1->0 after address and data are set.

DCS_CAL_DATA_IN[15:0]:   set via DCS write of address=24

DCS_CAL_ADDR[8:0]:   set via DCS write of address=25,  where bit[8]=1 if RD, 0 if WR

DCS_CAL_BUSY: status reported by bit[0] of DCS read of address=69

DCS_CAL_DATA_OUT[15:0]:   reported via DCS read of address=70 after BUSY is released

DCS_HV_INIT:  set to 1/0  via DCS write of address=26. Must toggle 1->0 after address and data are set.

DCS_HV_DATA_IN[15:0]:   set via DCS write of address=27

DCS_HV_ADDR[8:0]:   set via DCS write of address=28,  where bit[8]=1 if RD, 0 if WR

DCS_HV_BUSY: status reported by bit[4] of DCS read of address=69

DCS_CAL_DATA_OUT[15:0]:   reported via DCS read of address=71 after BUSY is released

Example 1: READ OF EVENT TAG in DIGI

Last local event tag seen by each DIGIs is kept in DIGI registers 0xD0 (bit 15:0), 0xD1 (bit 31:16) and 0xD2 (bit 47:32).  These registers can be read direct from the DIGI bypassing the uProc using the TWI protocol:

- Enable communication to TWI bypassing uProc

- Send the ADDR to read

- Latch the address by pulling INIT high and back low

- Check status of BUSY

- Read DATA_OUT

- Read reg = 6. If return 0, write to data = 1 to  reg = 6

read local counter bit[15:0] from CAL DIGI

- Write to CAL_ADDR via reg = 25  the value of 0x1D0  (NB: bit(9)=1 means TWI RD)

- Write to CAL_INIT via reg = 23 the value of  1, followed by 0

- Read CAL_BUSY status via bit(0) of reg = 69 and ensure that it is low

- Read CAL_DATA_OUT via reg = 70

Ex:  read address 0xD1 from HV DIGI

- Write to HV_ADDR via reg = 28  the value of 0x1D1  (NB: bit(9)=1 means TWI RD)

- Write to HV_INIT via reg = 26)  the value of 1, followed by 0

- Read HV_BUSY status via bit(4) of reg = 69 and ensure that it is low

- Read HV_DATA_OUT via reg = 71

NB: to prevent interfering with uProc commands, disable TWI by writing data = 0 to  reg = 6

Example 2: RESET OF DIGI FIFOs

The ROC can reset the DIGI FIFOs via TWI protocol, following the steps below:

- Read reg = 6. If return 0, write to data = 1 to  reg = 6

clear CAL DIGI FIFOs

- Write to CAL_ADDR via reg = 25  the value of 16  (NB: bit(8)=0 means TWI WR)

- Write to CAL_DATA_IN  via reg = 24 the value of 0

- Write to CAL_INIT  via reg = 23 the value of  1, followed by 0

- Write to CAL_DATA_IN  via reg = 24 the value of 1

- Write to CAL_INIT via reg = 23 the value of 1, followed by 0

clear HV DIGI FIFOs

- Write to HV_ADDR  via reg = 28  the value of 16  (NB: bit(8)=0 means TWI WR)

- Write to HV_DATA_IN  via reg = 27 the value of 0

- Write to HV_INIT  via reg = 26 the value of  1, followed by 0

- Write to HV_DATA_IN  via reg = 27 the value of 1

- Write HV CAL_INIT via reg = 26 the value of 1, followed by 0

NB: to prevent interfering with uProc commands, disable TWI by writing data = 0 to  reg = 6

Disabled features:

- Error counters read

Various errors (both diagnostic counters or FSM states) are sent to TOP_SERDES/ErrorCounter module (see list in “Error_counters_list.xlsx”) and can be read via:

- Write the error address to reg = 17

- Enable the error address via a write to reg=8 of bit(6)=1

- Read the error data via reg = 17

- Disable the error address via a write to reg=8 of bit(6)=0

Ex:  read number of DCS commands with CRC errors

- From “Error_counters_list.xlsx”, DCS CRC errors are recorded in error address=9

- Write 9 to reg=17

- Write 0x40 ( ie dec. 64) to reg=8  (even better, read reg=8, keep the original value and just add bit(6)=1)

- Read reg=17

- Write 0x0 to reg=8  (even better, write back the original value of reg. 8)

- DIGI HIT Simulation

One can set a programmable number of hits for each Serdes lane when sending internal patterns to the DDR (ie bypassing the DIGI data). This feature can be set by:

- Configure ROC to send patterns to DDR and enabling DIGI simulation by writing 0x810 (to send 320bit increasing counter) or 0x1810 (to send alternating 0x5555 and 0xAAAA) to address=8 (RW register)

- Set the number N of simulated DIGI hits per Serdes lane via a write to address=15 (RW register); default value is 2.

- Check on the status of the DIGI SIM FIFO via a DCS read of address 16: bit(11:8) return SIM_FIFOs empty; bit(7:4) return SIM_FIFOs full  (with LSB to MSB reporting status of CAL0, CAL1, HV0 and HV1 simulated Serdes, respectively)

Old features:

NEWRUN concept and HALTRUN_EN option: During data taking, a run can span multiple spills and it will be ended by the DTC sending a null HB. A null HB marks a window tag without a DATA REQUEST.  The NEWRUN gate is kept high by TOP_SERDES/RxPacketReader for the duration of the run. A NEWRUN rising edge is used to initialize and clear the ROC FIFO signals used in the DDR protocol and in the DTC protocol.

The NEWRUN edge is equivalent to DDR reset issued via a write of any value to reg=14, EXCEPT that the DDR address to write/read is not reset to zero as not to overwrite old data inside the DDR.

To simulate a HALT run situation in run control, the ROC firmware defines a HALTRUN_EN configuration bit (bit(13)=1 of reg=8) which, if set to 1, will prevent the clear of the heartbeat, event window and data request counters, as well as the 32-bit counters sent in pattern mode.

Ex: In DIGI data mode, if reg=8 is set equal to 0x30F, after a null HB all counters will be reset to zero. If reg=8 is set equal to 0x230F, the counters will keep incrementing after a null HB (unless an explicit DDR is sent)

The end of a run, and the ensuing null HB, is communicated to the DIGIs via the LAST_EWM gate, which envelops the event window marked following the NULL HB.

The DIGIs should respond by sending the data for the last window, disabling their digitization FIFO until the next non-null event marker is seen and resetting their EVENT TAG to zero.

To ensure the synchronization of tags sent from the ROC and seen the DIGI during the run we use:

- Read of event tag in DIGI at the end of the run

- TAG_SYNC signal from ROC to DIGI during the run every time the local event tag counter reached a multiple of 16385 (0x4000). If the DIGI find that their internal tag counters disagree, the pull bit(31)=1 of the hit header word.  The ROC monitors this bit and tag the event with an error if any SerDes report a TAG_SYN error. This info is saved in the first word of the event data header written to DDR (see DDR_Dump section of the header words definition), and in bit[5] of the STATUS word, passed to the Data Request Header.

Loopback logic:  the synchronization of the Mu2e DAQ is done by having a special LOOPBACK marker sent down the CF0->DTC chain, and measuring the time it takes to receive it back from each ROC. Based on the ROC with the longest latency, a delay in units of 5 ns is calculated for each ROC. The ROC should apply this as a delay to the Event Window Marker received by the DTC and the Event Window Marker using for its data digitization. The loopback delay is written via a DCS write to reg. 4. Note that at the present time the delay module (TOPSERDES/delay_sreq_1bit) can take a maximum delay of 31 counts.

Clock and Event Window controls via serial vs fiber: the ROC firmware allows to control which among fiber or serial commands can enable either the external (ie DTC) or internally generated signals to the DIGI, where by signals which mean either the 200 MHz CLK and the 5 ns Event Window Marker (EWM).

The following block diagram shows the logic involved in the generation of such signals. There are two cascades MUX, the first to set if fiber or serial are in charge, the second to choose between internal and external signals.

At power up, by default DIGIRW_SEL = 0, which means that the fiber (DCS_ENABLE) is in control of choosing between internal and external signals. Let’s not that for the 200 MHz CLK signal, while the internal CLK is always ON, the external CLK need to have the DTC fiber connected and locked; for EWM signal, the internal signal is generated by setting serial reg=0x81 high at a frequency set by reg=0x82 (in multiple of 5 ns), while the external EWM is set by the event window time interval.

The fiber controls the CLK signal via bit[8] of reg=0xB8 and the EWM signal via bit[9] of reg. 8. Their default value is low, which means that the “signal_enable” out of the first MUX is low and the internal signals are used to drive CLK and EWM to the DIGI. If bit[8] or bit[9] are high, then the external signals are used to drive CLK or EWM to the DIGI.

If we write DIGIRW_SEL = 1, which can be done ONLY via a write to serial reg=0xF2, then serial is in control of choosing between internal and external signals.

The serial controls the CLK signal via reg=0xB0 and the EWM signal via reg=0xB1. Their default value is low, which means that the “signal_enable” out of the first MUX is low and the internal signals are used to drive CLK and EWM to the DIGI. If either reg=0xB0 or reg=0xB1 are set high, then the external signals are used to drive CLK or EWM to the DIGI. Nore that DIGIRW_SEL = 1 will prevent taking DIGI data with the DTC and can be controlled ONLY by write to serial register 0xF2 or by a power cycle.

APPENDIX A

Change to roc firmware between “master” and “timingconstrained” branch (at the time of branch split). These changes were necessary to ensure that the ROC firmware meets timing, before and after moving from Libero2022.3 and Libero2024.2

- PF_NGMUX output drives two separate PF_CCC_C1 clock conditioning components, each in “post-divider” feedback mode driven by “output0 clock”, and with OUT0_FABCLK_0 driving the OUTBUF_DIFF to ROCtoHV/CAL_CLOCK0. Each PF_CCC_C1 drives the CLK and PLL_LOCK, and receives the PLL_POWERDOWN from a separate CORERESET (DigiReset_0 and DigiReset_1), whose EXT_RST_N is driven by SlowControls/DIGIDEVICE_RESETN via serial address 44. PF_CCC_C1 controlling ROCtoCAL clock also drives the CLK of EWMaker, counter_16bit (now unused), the DFN1 driving the EWM to the DIGIs, and DigiClkReset CORERESET driving the EWMaker reset (without any EXT_RST_N!)

- constraint/timing_pr.sdc: all constraints for hv_clk_out/cal_clk_out commented and replace by constraints on PF_CCC_C1 in constraint/ssync.sdc

- constraint/timing_user_constraints.sdc has separate asynchronous group definition (asyn3 to asyn21) for all of the clocks that used to be bunched up in asyn2 group, except for the PF_CCC_C1_0/1 output clocks which are in the same group as the TOP_SERDES/XCVR_Block/RX_CLK.

- constraint/timing_user_constraints.sdc also has some “set_false_path” for DCS_DDRESETN_N/CLK signal and “set_multicycle_path” for DCSProcessor/address_reg*/CLK signals

- PF_CCC_C0 still has four output clocks (200, 150, 80, 50 MHz) but the 200 MHz is now used ONLY for the CORESET PLL_LOCK/PLL_POWERDOWN, PF_NGMUX/CLK0 input and pulse_stretcher_1 (driven by DCS_DDRESET_N and used in EXT_RST_N logic to NewDDRInterface, DigiInterface and SloxControls). NOT USED any longer to drive DCS_CLK, replaced by 50 MHz clock from PF_CCC_C0.

- Added multiple CORERESET inside TOP_SERDES, for any reset to RXPacketFIFOs, DCSProcessor, DREQProcessor, and with EXT_RST_N driven either by DRACRegister/DCS_DDRRESET_N or by the combination of this signals with the timeout reset generated by the DCSProcessor and DREQProcessor themselves.

- TOP_SERDES/delay_sreg_1bit: max delay changed from 2048 (11 bit) to 32 (6 bit) clocks max

- TOP_SERDES/error_counter: disactivated

- TOP_SERDES/RxPacketReadxer: use registered event_marker_count in error checks. Separate TAG_LOST check in two steps. Register HB_TAG_SAVE+1 and SAVE_EVM_TAG+1 to use in error checks.

- TOP_SERDES/DREQ_Processor: make reset_dreq_logic negative logic. Add one state machine state before checking on register space using op_address.

- TOP_SERDES/DCS_Processor: make reset_dcs_logic negative logic.

- TOP_SERDES/DRACRegister: Rename HRESETN to POR_N. Rename EXT_RST_N to DDRReset_N, which is driven by CORERESET on DCS_CLK. Use DDRReset_N as a condition on the regular clock and not as a separate if-reset condition.

- TOP_SERDES/XCVR_Block: change ALIGN_RESET to DTC_ALIGN_RESET and and-with POR_N inside before connecting to ClockAligner RX_RESET_N. Replace driver of reset_align from ClockAligner/ALIGNMENT_RESET_N& ALIGN_RESET to CORERESET on RX_CLK with EXT_RST_N driven by  ClockAligner/ALIGNMENT_RESET_N&DTC_ALIGN_RESET.

- DIGIInterface/ROCFIFOController: register current_data to use in START2 state logic. Add extra state before START2 state and COUNT2 state.

- DDRInterface/EW_FIFO_Controller: register hb_tag_in  and dreq_tag_in to use in hb_tag_err and hb_dreq_err logic. Separate hb_tag_err accross two 32-bit wide checks (but not hb_dreq_err!)

- DDRInterface/ewtag_cntrl: add extra latching step in evm_on_serdesclk logic.

- SLOWCONTROLS/DCSRegister.vhd: changes to SENDWRITE state machine